Disclaimer:

This document was produced by DEQ. Some of its content may not be in an accessible format pursuant to Section 508 of the federal Rehabilitation Act of 1973, as amended (29 U.S.C. § 794 (d)). Please call 800-592-5482 if you need assistance.

Virginia Ambient Air Monitoring

2019 Annual Report

Department of Environmental Quality

Commonwealth of Virginia

Department of Environmental Quality

Office of Air Quality Monitoring

4949-C Cox Road

Glen Allen, VA 23060

This Ambient Air Monitoring Data Report is for the time period of January 1, 2019 to December 31, 2019.

David K. Paylor, Director

Virginia Department of Environmental Quality

Michael Dowd

Director, Division of Air Quality

Acknowledgements:

We would like to thank Chuck Turner, Carolyn Stevens, Dan Salkovitz, Baxter Gilley and Namita Verma.

Sunflower pictures were taken at Alvis Farms in Goochland, VA

Published, October 2020

2019 Annual Report prepared by:

Crystal Sorensen, Environmental Specialist

TABLE OF CONTENTS

GENERAL INFORMATION:

Introduction	 2

About Virginia’s Office of Air Quality Monitoring	 3

Frequently Asked Questions	 4

CRITERIA POLLUTANTS:

PM2.5  (Particulate Matter equal to or less than 2.5 microns)

Description	 8

PM10 (Particulate Matter equal to or less than 10 microns)

Description	 15

CO (Carbon Monoxide)

Description	 16

Graphs	 18

SO2 (Sulfur Dioxide)

Description	 20

Graphs	 23

NO2 (Nitrogen Dioxide)

Description	 26

Graphs	 29

OZONE

Description	 32

8-Hour Graphs	 37

LEAD

Description	 40

ACID RAIN:

Acid Precipitation Monitors	 43

VOLATILE ORGANIC COMPOUNDS (VOC):

Air Toxics Monitoring Network	 44

AQI (Air Quality Index)	 51

APPENDIX A:

Abbreviation Table	 60

Regional Site List and Monitoring Network	 61

Data Capture Criteria	 68

National Ambient Air Quality Standards	 69

Number of Criteria Pollutant Monitoring Sites	 70

Air Quality Planning Areas	 71

APPENDIX B:

Air Quality Internet Links	 73

References	 74

INTRODUCTION

The 2019 Virginia Ambient Air Monitoring Data Report is a compilation of air pollutant measurements made by the Virginia Department of Environmental Quality, the City of Alexandria, the U.S. Department of Agriculture - Forest Service, the National Park Service and the Environmental Protection Agency.  Ambient air quality was measured at 40 locations within the Commonwealth during 2019.  Included in this total are 1) three Sulfur Dioxide monitors that are maintained by three industrial companies that were identified in the recent EPA Data Requirements Rule and 2) two CASTNet sites operated by EPA.  All of these monitoring sites were established in accordance with EPA’s siting criteria contained in 40 CFR Part 58, Appendices D and E. Monitoring network operations conformed to EPA guidance documents and accepted air quality monitoring practices.  All data reported for these monitoring sites were quality assured in accordance with requirements contained in 40 CFR Part 58, Appendix A.  Ambient concentrations of carbon monoxide, nitrogen dioxide, sulfur dioxide and PM10 were within the EPA’s national ambient air quality standards (NAAQS) in 2019.  Virginia experienced a good ozone season in 2019.  There were four exceedance days in Northern Virginia and for the first time there were no days in the Richmond area where the standard of 0.070 ppm was exceeded.  For the 3-year period from 2017 through 2019 all ozone sites demonstrated compliance with the 0.070 ppm National Ambient Air Quality Standard (NAAQS) for ozone.  For 2019 there were no exceedances of the 35 micrograms per cubic meter (µg/m3) 24-hour standard for PM2.5 (particulate matter less the 2.5 microns).  The 2017 – 2019 design values for all sites in the Commonwealth for both the 24 hour and annual standard for PM2.5 are below the NAAQ standard.

Some significant changes that impacted the air quality monitoring network in 2019 are:

- 2019 represented the third year of data collection at the Data Requirements Rule (DRR) Sulfur Dioxide sites.  This is significant because the DRR provides for the possibility of shutdown of any DRR-required monitor after three years of operation if the data demonstrate a three-year design value of less than 50% of the standard.  AQM will review the data upon submittal and determine if the sources met this requirement.  This determination will then be sent to EPA for approval.

- EPA has proposed an extension of the start date for the PAMS requirements to be implemented at DEQ’s MathScience Innovation Center site.  Originally the start date was to be June 1, 2019 but due to nationwide issues with equipment acquisition and data evaluation and validation software EPA has published a proposal to extend the start date to June 1, 2021.  DEQ has received all required equipment and AQM projects that the new start date will be met.

- As part of the ongoing process of upgrading and expanding DEQ’s continuous PM2.5 monitoring capability the continuous instruments at our Roanoke (Vinton) site and our Fairfax (Lee Park) site were upgraded to newer instruments that employ broadband spectroscopy methodology.  These units are installed and are currently operating.

OFFICE OF AIR QUALITY MONITORING

AQM is responsible for seeing that the Virginia ambient air monitoring network is maintained and operated in accordance with state and federal guidelines. Personnel from the Department of Environmental Quality (DEQ) regional offices, the City of Alexandria, the National Park Service, and the U.S. Department of Agriculture Forest Service conduct the daily operations at these sites. One of AQM’s primary jobs is to support these people in their air quality monitoring efforts. AQM does this by:

- calibrating air monitoring instrumentation and associated support equipment on a set schedule

- auditing the instrumentation to insure that it is operating within set standards

- troubleshooting instrumentation problems reported by the regional operators

- supplying field operators with necessary items so they can perform their job properly

- repairing malfunctioning sampling instrumentation and ancillary equipment

Other functions:

- respond to regional and locality requests for special sampling such as emergency response or to answer citizen complaints

- coordinate efforts with the regional offices and localities to determine new air monitoring site locations

- conduct AQM generated special sampling projects to characterize a community's air quality

- furnish ambient air data to the regional offices, localities, Central Office, EPA and the EPA database

- answer FOIA requests for ambient air sampling data

- work with the regions and the localities to see that area monitoring needs are met

- work with EPA to see that necessary state and federal monitoring needs are met

- support SESARM (Southeastern States Air Resource Managers, Inc.) and MARAMA (Mid-Atlantic Regional Air Management Association of the Southeast) on routine and special projects

Criteria Pollutant Monitoring:

A portion of the air monitoring network is made up of instruments that sample for the Criteria Pollutants. Sulfur dioxide, nitrogen dioxide, carbon monoxide, ozone, lead and particulate matter (PM10 & PM2.5) can injure health, harm the environment and cause property damage. EPA calls these pollutants criteria air pollutants because they have regulated them by first developing health-based criteria (science-based guidelines) as the basis for setting permissible limits. One set of limits (primary standard) protects health; another set of limits (secondary standard) is intended to prevent environmental and property damage

Special Monitoring:

In addition to overseeing the air sampling network for criteria pollutants, AQM conducts routine and short term sampling for VOCs (volatile organic compounds), Carbonyls, Toxic Metals and NOy (total reactive nitrogen). Sampled VOCs are made up of 58 HAPs (Hazardous Air Pollutants) and 56 Hydrocarbon Ozone Precursors.

FREQUENTLY ASKED QUESTIONS

1.	What is the Clean Air Act?

The Clean Air Act is a federal law that provides for the protection of human health and the environment.  The original Clean Air Act was passed in 1963, and the 1970 version of the law resulted in the creation of the U.S. Environmental Protection Agency (EPA), which was charged with setting and enforcing ambient air quality standards.  The law was amended in 1977, and most recently in 1990.  Most of the activities of the Virginia Department of Environmental Quality’s Air Division come from mandates of the Clean Air Act, and are overseen by the EPA.  More information on the 1990 amendments to the Clean Air Act can be found at: Clean Air Act.

2.	What is a criteria air pollutant?

The Clean Air Act names six air pollutants that are commonly found in the air throughout the United States, and that can injure humans by causing respiratory and cardiovascular problems, and harm the environment by impairing visibility, and causing damage to animals, crops, vegetation and buildings.  EPA has developed health-based criteria for these pollutants through scientific studies, and has established regulations setting permissible levels of these pollutants in the air.  The “criteria” pollutants are:  carbon monoxide, sulfur dioxide, nitrogen dioxide, ozone, particulate matter, and lead, and the limits that have been set for them are the National Ambient Air Quality Standards (NAAQS).

3.	What is the difference between a primary and secondary National Ambient Air Quality Standard?

The National Ambient Air Quality Standards are divided into two types.  The first type, the primary standard, is designed to protect human health, especially those who are most vulnerable such as children and the elderly, and people suffering from asthma, emphysema, chronic bronchitis, and heart ailments.  The second type, the secondary standard, is designed to prevent damage to property and the environment.  For a list of the primary and secondary National Ambient Air Quality Standards, see NAAQS table or page 70 of this report.

4.	How is the location of an air monitoring station decided?

Generally, the deciding factor in all Virginia air monitoring sampling is to determine where the highest pollutant concentrations will occur, and place the sampler as near as possible to that location. A wind rose is typically used to determine the prevailing wind direction for an area and identify the downwind direction from a probable source.  A wind rose is a meteorological map showing the frequency and strength of winds from different directions at a specific location.

For typical criteria pollutant monitoring, the federal guidelines on siting an air monitor for measuring maximum concentrations are followed. These guidelines not only encourage siting in areas with free airflow and a minimum amount of obstructions, but they also give the height requirements for the sample inlet and the desired separation distances from obstructions such as tree lines, localized sources such as oil furnace flues, and other influences that can skew the data.

FREQUENTLY ASKED QUESTIONS

Other determining factors for placing air monitoring stations include:

- security of the site

- safety of the operator

- availability of electric power and communication service

- accessibility of the site

For more specific information, consult EPA's Quality Assurance Handbook for Air Pollution Measurement Systems, Volume II, Section 6, EPA's Quality Assurance Handbook

5.	How large of an area does an air monitoring station represent?

The sampling area of a monitoring site is dependent on the parameters selected for representation, such as:

- type of pollutants being sampled

- rural vs. urban sampling

- source oriented, population oriented, or background oriented

- sampling for pollution transported from outside the Commonwealth

Many sites are also dependent on topography and meteorology of an area, which play an important role. Federal guidelines spell out the general area of representation. Some examples of varied air sampling sites are:

- A background research site in central Virginia may represent an area with a radius of 50 to 100 kilometers.

- An ozone or fine particulate site in the Shenandoah Valley may represent an elongated area with an axis running with the valley and is a hundred kilometers long but only twenty-five kilometers wide.

- A carbon monoxide sampling site in an urban street canyon setting may represent an area of only a few blocks in radius.

- A source oriented site in south central Virginia may represent an area from 0.5 to 4 kilometers in radius.

For more specific information, consult EPA's Quality Assurance Handbook for Air Pollution Measurement Systems, Volume II, Section 6, EPA's Quality Assurance Handbook

6.	What is a “nonattainment” area?

A nonattainment area is a geographic area that has been determined by EPA as not meeting the air quality standards for one or more pollutants.  Typically, an area is declared nonattainment based on data collected at one or more ambient air monitoring sites within the area.  However, sometimes the nonattainment designation can be made based on the use of air quality models that use monitoring data from other areas.  In Virginia, nonattainment areas are designated for ozone.

7.	How can I find out if I live in a nonattainment area?

A list of nonattainment areas in Virginia can be found in this report on page 74.  EPA has a list of all nonattainment areas in the country at Green Book National Area and County-Level Multi-Pollutant Information.

FREQUENTLY ASKED QUESTIONS

8.	What are the impacts of a nonattainment designation?

To demonstrate how they plan to achieve federal air quality standards, states must draft a “State Implementation Plan,” or SIP.  This plan lists specific actions that the state will undertake to improve and maintain acceptable air quality, and a time frame for accomplishing these goals.  The SIP may require new factories to install the newest and most effective air pollution control technologies.  Other actions could be requiring older factories to retrofit their smokestacks with better pollution control devices, requiring an area to sell only reformulated gasoline during the summer months, requiring vapor recovery systems on gasoline pumps, and requiring vehicle exhaust emission checks, to name a few.  SIP development is a lengthy process, and involves negotiation between the state and the EPA until it is finalized.

9.	What is a Maintenance Area?

A maintenance area is an area that has formerly been designated nonattainment, but is now recognized by EPA as meeting the NAAQS.  A maintenance area must have an approved “maintenance plan” to meet and maintain air quality standards.

10.	What is a design value?

A design value is a statistic that describes the air quality status of a given area relative to the level of the National Ambient Air Quality Standards (NAAQS).  Design values are typically used to classify nonattainment areas, assess progress towards meeting the NAAQS, and develop control strategies.  Design values are expressed as concentrations in the ambient air and are calculated according to regulatory specifications to determine the highest monitored concentration in an attainment or non-attainment area.

11.	How can I get current or historical air quality data?

Current AQI and air quality forecasts can be obtained from EPA's AirNow website.  Summary air quality data for PM2.5 can also be found at Air Monitoring Particulate summary.  Prior annual monitoring data reports can be found at Air Monitoring Publications.  EPA also provides monitoring and emissions data, as well as maps, on the web at their AirData and Air Emissions Inventories webpages.  Detailed data for monitoring sites in Virginia can also be obtained by contacting the VA DEQ Office of Air Quality Monitoring, or from EPA’s AQS Data Mart.

12.	What do I do if I have a complaint about air quality in my neighborhood?

Contact the DEQ regional office in your area by phone, or make a report online.  For a list of regional offices and phone numbers, see page 62 of this report, or visit DEQ's locations.

13.	Who can I call about an indoor air quality problem, such as mold or radon gas?

Your local health department may be able to assist you with some indoor air quality problems.  See Virginia Department of Health Local Districts for the health department office in your area or the Division of Environmental Epidemiology at (804) 864-8182.  The DEQ has an Indoor Air Quality webpage with links to websites that provide information regarding specific indoor air quality problems.  Other excellent sources of information on indoor air quality can be found on EPA's Indoor Air Quality website and through the American Lung Association website at LungUSA.

CRITERIA POLLUTANTS

PM2.5 Monitoring Network

PM2.5 is particulate matter (PM) that is less than or equal to 2.5 micrometers (a micrometer is one millionth of a meter) in aerodynamic diameter.  These particles are often called “fine particles” because of their small size.  Fine particles originate from a variety of man-made stationary and mobile sources, such as factory smoke stacks and diesel engines, as well as from natural sources, such as forest fires and dust storms.  These particles may be emitted directly into the air, or they may be formed by chemical reaction in the atmosphere from gaseous emissions of sulfur dioxide (SO2), nitrogen oxides (NOx), and volatile organic compounds (VOCs) and other chemicals.

Scientific research has linked fine particle pollution to human health problems.  The particles are easily inhaled deep into the lungs, and can actually enter the bloodstream.  Particle pollution is of particular concern to people with heart or lung disease, such as coronary artery disease, congestive heart failure, asthma, or chronic obstructive pulmonary disease (COPD).  Older adults are at risk because they may have underlying, undiagnosed heart or lung problems.  Young children are also at risk because their lungs are still developing, they are more likely to have asthma or acute respiratory disease, and they tend to spend longer periods of time at high activity levels, causing them to inhale more particles than someone at rest.  Even otherwise healthy people may suffer short-term symptoms such as eye, nose, throat irritation, coughing, and shortness of breath during episodes of high particulate levels.

PM2.5 air quality standards were implemented by EPA in 1997 to protect against the health effects of fine particle pollution.  On December 14, 2012, EPA strengthened the nation’s air quality standards for fine particle pollution to improve public health protection by revising the primary annual PM2.5 standard to 12.0 g/m3 and retaining the 24-hour fine particle standard of 35 g/m3.  For more information, see EPA's Revisions to the AQ Standards for Particulate Matter.

In addition to health problems, fine particle pollution contributes to haze that causes deterioration of visibility in scenic areas, and also deposits harmful compounds on the soil and water.  Unlike ozone, which is a seasonal pollutant in most areas of the country, particle pollution can occur year-round, and is monitored throughout the year in Virginia.  The Virginia DEQ PM2.5 monitoring network uses three different types of samplers to monitor fine particulate in the state:

PM2.5 24-hour Mass Sampler:  This Federal Reference Method (FRM) sampler collects particulate matter on a stretched Teflon filter media.  The samplers collect 24-hour samples on a one-in-three day schedule, except for one in Fairfax County, which collects samples every day.  The 3-day monitoring schedule can be found at PM2.5 Monitoring Calendar.  The filters are retrieved from the field and shipped via courier to the Virginia Division of Consolidated Laboratory Services (DCLS) in Richmond.  At the laboratory, the filters are equilibrated for a minimum of 24 hours prior to the final weighing.

PM2.5 Monitoring Network

PM2.5 24-hour Speciation:  Speciated PM2.5 data are collected at one site in Virginia, the MathScience Innovation Center in Henrico Co., using two co-located samplers that operate simultaneously.  One sampler, the MetOne Super SASS, collects particulate matter on two filters, one nylon and the other Teflon.  The second sampler, the URG 3000N carbon sampler, uses a quartz filter to collect particulate matter.  The samplers run for 24 hours, on a one-in-three day sampling schedule.  After the completion of a sample run, the instrument operator removes the exposed filters and ships them via refrigerated container to an EPA contract lab, where the filters are analyzed for the following:

- Teflon filter:  thirty-three trace elements including aluminum, antimony, arsenic, bromine, calcium, iron, lead, silicon, titanium, vanadium, and zirconium

- Nylon filter:  cations (ammonium, potassium, sodium) and anions (nitrate, sulfate chloride)

- Quartz filter:  carbons (elemental carbon, organic carbon, total carbon)

The resulting data provide a “chemical fingerprint” of air masses moving through the Richmond area.  These data, in conjunction with historical data from other speciation sites, including those outside Virginia, give a representative picture of the constituents of the air samples, which help identify sources of high values and show how the air masses move over a broad area.

PM2.5 Continuous:  Unlike the PM2.5 24-hour sampler, these instruments collect particulate samples on a continuous basis, and data can be compiled into hourly and 24-hour averages.  PM2.5 continuous samplers are operated in Henrico Co., Roanoke Co., Fairfax Co., Shenandoah National Park, Frederick Co., Albemarle Co., and the Cities of Richmond and Hampton.  Some of these continuous monitors are designated as Federal Equivalent Method (FEM) monitors, meaning that they may be used to determine NAAQS compliance as well as to report the AQI, whereas the TEOM continuous monitors are used strictly for AQI and forecasting purposes only.

The purpose of each of the PM2.5 samplers is determined by the type and designation of the sampler.  The FRM and the FEM samplers collect data that are used to determine if the state is complying with the national ambient air quality standards (NAAQS) for particulate matter.  The speciation sampler collects data about the composition of particulate matter in Virginia, and is useful for identifying potential sources of air pollution both within and outside the state boundaries.  The FRM and speciation monitors are manual, filter-based methods, and the samples they collect must be transported to a laboratory for processing.  Consequently, they are not useful for reporting real-time air quality conditions.  The continuous particulate monitors provide real-time data on fine particulate levels.  The hourly average data are polled by a central computer, and then posted on the DEQ Air Monitoring webpage.  The data are also simultaneously sent to EPA’s national air quality website at Airnow.

PM2.5 Monitoring Network

In addition to the PM2.5 network operated by the DEQ, the National Park Service and the USDA Forest Service operate PM2.5 samplers at Big Meadows in Shenandoah

National Park, and in Rockbridge Co. as part of the IMPROVE (Interagency Monitoring of Protected Visual Environments) network.  This network employs different sampling

methods than those used by the DEQ.  Data for the IMPROVE network can be found on the internet at Improve Network.

PM2.5 Monitoring Network

PM2.5 Monitoring Network

National Ambient Air Quality Standards (NAAQS)

Primary Standard for PM2.5:

- Annual Arithmetic Mean – the 3 year average of the weighted annual mean PM2.5 concentration must not exceed 12.0 g/m3.

- 24-Hour concentration – the 3 year average of the 98th percentile of 24-hour concentrations must not exceed 35 g/m3.

Secondary Standard for PM2.5:

- Annual Arithmetic Mean – the 3 year average of the weighted annual mean PM2.5 concentration must not exceed 15.0 g/m3.

| 2017-2019 PM2.5 24-hour Averages, 98th Percentile Values (g/m3, LC) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Site | 2017 | 2018 | 2019 | 3-Year Average |
| (101-E) Bristol | 16.2 | 14.3 | 13.4 | 15 |
| (26-F) Rockingham Co. | 14.6 | 18.4 | 17.8 | 17 |
| (28-J) Frederick Co. | 17.1 | 22.6 | 22.8 | 21 |
| (33-A) Albemarle Co. | 13.6 | 13.8 | 13.3 | 14 |
| (19-A6) Roanoke Co. | 14.2 | 15.6* | 12.9 | 14 |
| (110-C) Salem | 14.0 | 15.3 | 13.2 | 14 |
| (155-Q) Lynchburg | 14.6 | 13.5 | 11.6 | 13 |
| (71-H) Chesterfield Co. | 14.6 | 14.1 | 12.4 | 14 |
| (72-M) Henrico Co. | 14.5 | 16.2 | 14.4 | 15 |
| (72-N) Henrico Co. | 14.7 | 14.5 | 13.2 | 14 |
| (75-B) Charles City Co. | 13.2 | 13.7 | 13.8 | 14 |
| (158-X) Richmond | 17.0* | 19.4 | 19.7 | 19 |
| (179-K) Hampton | 15.5 | 13.4 | 13.8 | 14 |
| (181-A1) Norfolk | 12.7 | 13.8 | 16.9 | 14 |
| (184-J) Va. Beach | 13.5* | 14.6* | 17.5 | 15 |
| (38-I) Loudoun Co. | 14.2 | 16.6 | 19.8 | 17 |
| (47-T) Arlington Co. | 16.2 | 15.9 | 19.2 | 17 |
| (46-B9) Franconia, Fairfax Co. | 15.1 | 16.5 | 20.3 | 17 |
| (46-C2) Springfield, Fairfax Co. | 19.2* | 19.7 | 25.2 | 21 |

* Annual value did not meet completeness criteria

PM2.5 Monitoring Network

NAAQS

Primary Standard for PM2.5:

- Annual Arithmetic Mean – the 3 year average of the weighted annual mean PM2.5 concentration must not exceed 12.0 g/m3.

- 24-Hour concentration – the 3 year average of the 98th percentile of 24-hour concentrations must not exceed 35 g/m3.

Secondary Standard for PM2.5:

- Annual Arithmetic Mean – the 3 year average of the weighted annual mean PM2.5 concentration must not exceed 15.0 g/m3.

| 2017-2019 PM2.5 Weighted Annual Arithmetic Means (g/m3, LC) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Site | 2017 | 2018 | 2019 | 3-Year Average |
| (101-E) Bristol | 7.2 | 6.7 | 6.4 | 6.8 |
| (26-F) Rockingham Co. | 6.9 | 6.8 | 7.1 | 6.9 |
| (28-J) Frederick Co. | 7.5 | 7.3 | 7.6 | 7.4 |
| (33-A) Albemarle Co. | 6.7 | 6.0 | 6.4 | 6.4 |
| (19-A6) Roanoke Co. | 6.6 | 6.6* | 6.3 | 6.5 |
| (110-C) Salem | 7.0 | 7.0 | 6.6 | 6.9 |
| (155-Q) Lynchburg | 6.5 | 6.4 | 6.2 | 6.4 |
| (71-H) Chesterfield Co. | 7.1 | 6.5 | 6.7 | 6.8 |
| (72-M) Henrico Co. | 7.5 | 6.9 | 6.9 | 7.1 |
| (72-N) Henrico Co. | 7.2 | 6.7 | 6.8 | 6.9 |
| (75-B) Charles City Co. | 6.9 | 6.4 | 6.7 | 6.7 |
| (158-X) Richmond | 8.4* | 8.2 | 8.4 | 8.3 |
| (179-K) Hampton | 6.6 | 6.0 | 6.3 | 6.3 |
| (181-A1) Norfolk | 6.9 | 6.6 | 7.1 | 6.9 |
| (184-J) Va. Beach | 6.8* | 6.7* | 6.8 | 6.8 |
| (38-I) Loudoun Co. | 7.3 | 7.0 | 7.2 | 7.2 |
| (47-T) Arlington Co. | 7.6 | 7.4 | 8.0 | 7.7 |
| (46-B9) Franconia, Fairfax Co. | 6.9 | 6.9 | 7.2 | 7.0 |
| (46-C2) Springfield, Fairfax Co. | 9.0* | 8.9 | 9.1 | 9.0 |

* Annual value did not meet completeness criteria.

PM10 Monitoring Network

PM10 is particulate matter comprised of solid particles or liquid droplets with an aerodynamic diameter of less than or equal to 10 micrometers, and is sometimes referred to as “coarse particles.”  PM10 particles are larger than PM2.5, but are still in a size range that can pose health problems because they can be inhaled, and retained in the human respiratory system, causing breathing difficulties, and eye, nose and throat irritation.  In addition to the health effects of PM10, these particles can impair visibility, can contribute to climate change, and result in “acidic dry deposition.”  Acidic dry deposition occurs when particles containing acidic compounds fall to the ground.  The acidic particles can corrode surfaces that they settle on, and can increase the acidity of the soil and water.

The National Ambient Air Quality Standards, or NAAQS, for particulate matter were revised in September 2006.  EPA changed the existing standards for PM10 by revoking the annual standard of 50 micrograms per cubic meter, because current scientific evidence did not support a link between long-term exposure to coarse particles and health problems.  However, the 24-hour PM10 standard was retained to protect citizens from effects of short-term exposures.  For additional information on the revised particulate matter standards, see PM10 Standards.

To measure PM10, ambient air is drawn into a sampler that uses a particle size discrimination inlet.  The inlet is designed so that particles in the size range of 10 micrometers (also called microns) or below stay suspended in the air stream, while larger particles settle out.  The particles are captured on a filter, which is weighed before and after sampling, and the PM10 concentration is determined by dividing the change in filter mass by the volume of sampled air.  The resulting PM10 concentration is reported as micrograms per cubic meter (g/m3).  The normal sampling schedule is once every sixth day from midnight to midnight.  The 6-day monitoring schedule can be found at PM10 Monitoring calendar.

PM10 Monitoring Network

National Ambient Air Quality Standards (NAAQS)

Primary Standard for PM10:

- 24- hour concentration not to exceed 150 g/m3 more than once per year averaged over three years.  An exceedance means a 24-hour average value that is above the level of the 24-hour standard after rounding to the nearest 10 g/m3.

Secondary Standard for PM10:

- Same as Primary.

| 2017-2019 PM10 24-Hour Average Concentrations (units in g/m3 STD ) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Site | 2017 |  | 2018 |  | 2019 |  | >150 g/m3 |
|  | 1st Max | 2nd Max | 1st Max | 2nd Max | 1st Max | 2nd Max |  |
| (23-A) Carroll Co. | 21 | 19 | 24 | 22 | 19 | 18 | 0 |
| (134-C) Winchester | 20 | 20 | 22 | 20 | 26 | 25 | 0 |
| (72-M) Henrico Co. | 25 | 22 | 19 | 19 | 23 | 19 | 0 |
| (154-M) Hopewell | 20 | 19 | 20 | 18 | 20 | 17 | 0 |
| (179-K) Hampton | 19 | 19 | 21 | 17 | 19 | 18 | 0 |
| (181-A1) Norfolk | 21 | 20 | 22 | 20 | 20 | 20 | 0 |
| (44-A) Stafford Co. | -- | -- | 23 | 23 | 17 | 16 | 0 |
| (46-B9) Fairfax Co. | 30 | 27 | 24 | 21 | 20 | 19 | 0 |

CO Monitoring Network

Carbon monoxide (CO) is a colorless, odorless gas that is produced by incomplete burning of carbon compounds in fossil fuels (gasoline, natural gas, coal, oil, etc.).  Over half of the CO emissions in the country come from motor vehicle exhaust.  Other sources include construction equipment, boats, lawnmowers, woodstoves, forest fires, and industrial manufacturing processes.

CO concentrations are higher in the vicinity of heavily traveled highways, and drop rapidly the further the distance from the road.  Ambient levels of carbon monoxide tend to be higher in the colder months due to “thermal inversions” that trap pollutants close to the ground.  A thermal inversion occurs when the temperature of the air next to the ground is colder than air above it.  When this happens, the air resists vertical mixing that can help the pollutants to disperse, forming a layer of smog close to the ground.

Carbon monoxide is harmful because it reacts in the bloodstream, reducing the amount of oxygen that is supplied to the heart and brain.  CO can be harmful at lower levels to people who suffer from cardiovascular disease, like angina, arteriosclerosis, or congestive heart failure.   At high levels, CO can impair brain function, causing vision problems, reduce manual dexterity, and reduce ability to perform complicated tasks.  At very high levels, carbon monoxide can be deadly.

Carbon monoxide in the ambient air is measured continuously with an electronic instrument that uses NDIR, “non-dispersive infrared” photometry.  The instrument has a pump that continuously draws air through a sample chamber that contains an infrared light source and a detector.  Any CO molecules that are present in the sample air absorb some of the infrared light, reducing the intensity of the light reaching the detector.  The portion of the infrared light absorbed by the CO molecules is converted into an electrical signal corresponding to the CO concentration, and stored in the instrument computer.

CO Monitoring Network

National Ambient Air Quality Standards (NAAQS)

Primary Standard for CO:

- 8-hour average not to exceed 9 ppm more than once per year.

- 1-hour average not to exceed 35 ppm more than once per year.

There are no Secondary Standards for CO because it does not harm vegetation or buildings.

| Site | 2019 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1-Hour Avg. (ppm) |  | 8-Hour Avg. (ppm) |  |
|  | 1st Max. | 2nd Max. | 1st Max. | 2nd Max. |
| (19-A6) Roanoke Co. | 0.8 | 0.7 | 0.6 | 0.6 |
| (72-M) Henrico Co. | 1.1 | 1.1 | 1.1 | 0.9 |
| (158-X) Richmond | 1.7 | 1.7 | 1.5 | 1.2 |
| (179-K) Hampton | 0.7 | 0.7 | 0.6 | 0.5 |
| (181-A1) Norfolk | 1.5 | 1.4 | 1.2 | 1.0 |
| (46-C2) Fairfax Co. | 1.5 | 1.4 | 1.2 | 1.1 |
| (47-T) Arlington Co. | 1.7 | 1.7 | 1.3 | 1.3 |

Eight Hour Averages stated as Ending Hour

SO2 Monitoring Network

Sulfur dioxide (SO2) is a colorless gas that has a strong odor.  It results from burning of fuels containing sulfur (such as coal and oil), petroleum refining, and smelting (extracting metals from ore), and it also occurs naturally from volcanic eruptions.  SO2 can dissolve in water vapor to produce sulfuric acid, and it can also interact with other gases and particles in the air to produce sulfate aerosols that are capable of traveling long distances in the atmosphere.

EPA has developed primary and secondary air quality standards for SO2.  The primary standards are designed to protect people from the health effects of sulfur dioxide gas, which include respiratory problems for people with asthma and for those who are active outdoors.  Long-term exposure to high concentrations of sulfur dioxide gas can cause respiratory illness and aggravate existing heart conditions.  Sulfate particles that are formed from SO2 gas can be inhaled, and are associated with increased respiratory symptoms and disease.

Secondary standards for sulfur dioxide protect against damage to vegetation and buildings, and against decreased visibility.  The acids that can form from SO2 and water vapor contribute to acid deposition (commonly called “acid rain”) which causes damage to the leaves of plants and trees, making them vulnerable to disease, and can increase the acidity of lakes and streams, making them unsuitable for aquatic life.  Acid deposition also causes deterioration of materials on buildings, monuments, and sculptures.  Finally, small sulfate particles, formed when SO2 gas reacts with other gases and particles in the air, contribute to haze that causes decreased visibility in many areas of the country.

Sulfur dioxide is monitored continuously with an electronic instrument using ultraviolet fluorescence detection.  The instrument has a pump that pulls outside air into a sample chamber containing a high intensity ultraviolet (UV) light.  Any SO2 molecules in the sample air absorb some of the UV light, become excited, and then fluoresce, releasing light characteristic of SO2.  The fluorescence is detected with a photomultiplier tube (a tube that detects very small amounts of light and multiplies the signal many times), and the resulting signal, which corresponds to the amount of SO2 in the sample, is converted to an SO2 concentration by the instrument computer.

SO2 Monitoring Network

National Ambient Air Quality Standards (NAAQS)

Primary Standard for SO2:

- 3-year average of the 99th percentile 1-hour daily maximum values not to exceed 75 ppb.

Secondary Standard for SO2:

- 3-Hour concentration not to exceed 0.5 ppm (500 ppb) more than once per year.

| Sulfur Dioxide 99th Percentile 1-Hour Daily Maximum Values (ppb) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Site ID | City/County | 2017 | 2018 | 2019 | 3-Yr Avg Design Value 2016-2018 |
| 26-F | Rockingham Co. | 2 | 2 | 2 | 2 |
| 19-A6 | Roanoke Co. | 3 | 2 | 3 | 3 |
| 72-M | Henrico Co. | 4 | 5 | 3 | 4 |
| 75-B | Charles City Co. | 18 | 18 | 14 | 16 |
| 179-K | Hampton | 3 | 21 | 3 | 9 |
| 181-A1 | Norfolk | 3 | 10 | 2 | 5 |
| 46-B9 | Fairfax Co. | 4 | 5 | 4 | 4 |

SO2 Monitoring Network

National Ambient Air Quality Standards (NAAQS)

Primary Standards for SO2:

- 3-year average of the 99th percentile 1-hour daily maximum values not to exceed 75 ppb.

Secondary Standard for SO2:

- 3-Hour concentration not to exceed 0.5 ppm (500 ppb) more than once per year.

| Sulfur Dioxide 3-Hour Block Average Maximum Values (ppb) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Site ID | City/County | 2017 | 2018 | 2019 | Number Obs. > 500 ppb |
| 26-F | Rockingham Co. | 2 | 2 | 2 | 0 |
| 19-A6 | Roanoke Co. | 3 | 3 | 3 | 0 |
| 72-M | Henrico Co. | 5 | 5 | 2 | 0 |
| 75-B | Charles City Co. | 15 | 15 | 14 | 0 |
| 179-K | Hampton | 3 | 24 | 2 | 0 |
| 181-A1 | Norfolk | 5 | 18 | 3 | 0 |
| 46-B9 | Fairfax Co. | 5 | 6 | 5 | 0 |

NO2 Monitoring Network

Nitrogen dioxide (NO2) is one in a group of gases referred to as oxides of nitrogen (NOX).  Nitrogen dioxide, which is characterized by a reddish-brown color and pungent odor, along with the other NOX gases, results from high-temperature burning of fossil fuels in automobiles, power plants, and other industrial, commercial, and residential sources.  NOX can occur naturally from lightning, forest fires, and bacterial processes that take place in soil.

NOX pollution contributes to a wide range of problems in the environment.  Ground-level ozone, a major component of “smog”, forms when NOX and volatile organic compounds (VOCs) react in the presence of sunlight.  NOX also reacts with other gases and particles in the air to form acids that contribute to acid deposition, and to form small particles that can be inhaled into the lungs.  NOX contributes to water quality deterioration by depositing nitrogen into water bodies, upsetting the nutrient balance and causing oxygen depletion in the water so that fish and other aquatic life cannot survive.  Nitrate particles and nitrogen dioxide also contribute to visibility impairment by blocking light transmission.

EPA has established primary and secondary air quality standards for NO2 because it can cause lung irritation and respiratory problems in humans.  Small particles formed from reaction of NOX gases with other compounds can be inhaled deep into the lungs and cause or worsen respiratory conditions such as emphysema and bronchitis, and can aggravate existing heart conditions.

Nitrogen oxides are measured continuously with electronic instruments using the “gas phase chemiluminescence” method.  The instrument has a pump that draws ambient air into a reaction chamber.  Inside the chamber, the air is mixed with a high concentration of ozone (O3). Any nitric oxide (NO) present in the sample air reacts with O3 to produce NO2.  The NO2 molecules created by the reaction are in an excited state, and emit light characteristic of NO2 – this is called “chemiluminescence.”  The light produced in the reaction is detected with a photomultiplier tube, and the resulting signal is converted to a number reflecting the concentration of NO in the ambient air by the instrument computer.  The instrument then activates a valve that diverts incoming ambient air into a “converter”, which converts any NO2 in the ambient air to NO by reduction reaction.  After the air passes through the converter, it is sent to the reaction chamber where the NO and O3 react to produce NO2.  The chemiluminescence produced by the reaction is converted to a signal that reflects the concentration of NOX in the ambient air.  The instrument then calculates the NO2 concentration using the difference between the measured NOx and NO concentrations.

NO2 Monitoring Network

National Ambient Air Quality Standards (NAAQS)

Primary Standard for NO2:

- 3-year average of the 98th percentile 1-hour daily maximum values not to exceed 100 ppb.

- Annual Arithmetic Mean not to exceed 53 ppb (.053 ppm).

Secondary Standard for NO2:

- Annual Arithmetic Mean not to exceed 53 ppb (.053 ppm).

| Nitrogen Dioxide 98th Percentile 1-Hour Daily Maximum Values (ppb) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Site City/County | 2017 | 2018 | 2019 | 3-Yr Avg. Design Value 2017-2019 |
| (26-F) Rockingham Co. | 34.7 | 35.6 | 37.5 | 36 |
| (19-A6) Roanoke Co. | 31.9 | 30.8 | 32.3 | 32 |
| (72-M) Henrico Co. | 38.8 | 31.4 | 30.8 | 34 |
| (75-B) Charles City Co. | 32.2 | 30.5 | 31.1 | 31 |
| (158-X) Richmond | 42.5 | 43.1 | 42.6 | 43 |
| (179-K) Hampton | 28.1 | 25.5 | 24.2 | 26 |
| (181-A1) Norfolk | 40.2 | 38.4 | 39.6 | 39 |
| (38-I) Loudoun Co. | 35.0 | 32.6 | 37.5 | 35 |
| (45-L) Prince William Co. | 24.0 | 21.0 | 24.3 | 23 |
| (46-C2) Fairfax Co. | 44.3 | 46.6 | 48.1 | 46 |
| (47-T) Arlington Co. | 42.5 | 41.9 | 44.8 | 43 |

NO2 Monitoring Network

National Ambient Air Quality Standards (NAAQS)

Primary Standard for NO2:

- 3-year average of the 98th percentile 1-hour daily maximum values not to exceed 100 ppb.

- Annual Arithmetic Mean not to exceed 53 ppb (.053 ppm).

Secondary Standard for NO2:

- Annual Arithmetic Mean not to exceed 53 ppb (.053 ppm).

| Site | Annual Arithmetic Mean (ppb) |  |  |
| --- | --- | --- | --- |
|  | 2017 | 2018 | 2019 |
| (26-F) Rockingham Co. | 8 | 8 | 9 |
| (19-A6) Roanoke Co. | 5 | 5 | 5 |
| (72-M) Henrico Co. | 8 | 6 | 6 |
| (75-B) Charles City Co. | 4 | 5 | 5 |
| (158-X) Richmond | 12 | 12 | 12 |
| (179-K) Hampton | 4 | 3 | 3 |
| (181-A1) Norfolk | 7 | 7 | 8 |
| (38-I) Loudoun Co. | 6 | 6 | 7 |
| (45-L) Prince William Co. | 4 | 4 | 4 |
| (46-C2) Fairfax Co. | 15 | 15 | 16 |
| (47-T) Arlington Co. | 9 | 9 | 10 |

Ozone Monitoring Network

Ozone (O3) is a gas comprised of three oxygen atoms.  It is unstable, and a strong oxidizing agent, and will react readily with other compounds to decay to the more stable diatomic oxygen (O2).

Ozone is a gas that occurs both in the Earth's upper atmosphere and at ground level. Ozone can be "good" or "bad" for people's health and for the environment, depending on its location in the atmosphere.  “Good” ozone occurs naturally in the stratosphere, about 10-30 miles above the earth’s surface, where it forms a layer that filters the sun’s ultraviolet rays before they reach the surface where they can cause harm to people, animals, and plants.  “Bad” ozone, or ground-level ozone, occurs when chemicals found in the atmosphere at earth’s surface react in the presence of intense sunlight.  Ozone at ground level is harmful because it can cause a variety of health problems, as well as damage to plants and materials.  Since ground-level ozone is not emitted directly, it is called a “secondary” pollutant.  The chemicals needed to form ozone, NOx and hydrocarbons (also called volatile organic compounds, or VOCs), can come from motor vehicle exhaust, power plants, industrial emissions, gasoline vapors, chemical solvents, as well as natural sources such as lightning, forest fires, and plant decomposition.  Ozone, and the chemicals that produce ozone, can travel hundreds of miles from their sources, so that even rural areas with few pollutant emissions can occasionally experience high ozone levels.  Efforts to control ground-level ozone involve limiting emissions of NOx and VOCs, or “ozone precursors,” that are necessary for ozone production.

Ground-level ozone is a seasonal pollutant, and the length of the ozone season varies across the country.  In some areas, the season may last most of the year, but in Virginia it is usually only a problem during the late spring to summer months when the sunlight is most intense.  Virginia is required to operate its ozone monitors from the months of March to October.  In addition to the seasonal pattern, ozone also has a strong diurnal (daily) pattern at low altitudes, so that it is usually depressed at night, but begins to build during the day after the sun rises.

EPA has created primary and secondary air quality standards for ground-level ozone because of its adverse effects on public health and welfare.  In humans, ozone can irritate lung airways, causing sunburn-like inflammation, and can induce symptoms such as wheezing, coughing, and pain when taking a deep breath.  Although people with existing respiratory problems, such as asthma and emphysema, are most vulnerable, young children and otherwise healthy people can also suffer respiratory problems from ozone exposure.  Scientific studies have shown that even at low levels, ozone can trigger breathing problems for sensitive individuals.  In addition to human health problems, ozone can damage the leaves of plants and trees, making them susceptible to disease, insects, and harsh weather.  Ozone can also cause rubber to harden and crack, and some painted surfaces to fade more quickly.

Ozone Monitoring Network

Ozone is measured continuously with electronic instruments using “ultraviolet (UV) absorption photometry.”  The method is based on the principle that ozone strongly absorbs UV light at 254 nanometers (a nanometer is equal to a distance of one billionth of a meter).  The ozone monitor has a sample pump that draws ambient air into it and splits the air into two gas streams.  In one stream, the air passes through an “ozone scrubber”, which cleanses the sample air of any ozone.  Then the clean air passes through a sample cell that contains a UV light source and a detector.  The detector measures the intensity of the light in the sample cell, providing a zero reference.  The second air stream is sent straight into the sample cell, bypassing the scrubber.  Any ozone present in the incoming air will absorb some of the UV light in the sample cell, reducing the amount of light reaching the detector.  The instrument then calculates the ozone concentration of the ambient air from the difference in the light intensities measured between the scrubbed, or “zero” air, and the unscrubbed air.

Daily ozone forecasts for selected metropolitan areas and hourly ozone values for all Virginia ozone monitoring sites can be viewed for the months of March to October on the DEQ web page.  In addition, animated ozone maps for Virginia and other parts of the United States are available at EPA's AirNow website.

Ozone Monitoring Network

The National Park Service operated one ozone monitor at Big Meadows in Shenandoah National Park in 2019.  Daily data from this site are available at the DEQ website, and historical data may be obtained on the internet at NPS Gaseous Pollutant and Meteorological Data or at EPA’s AirData website.  EPA also maintains ozone monitoring sites in Giles and Prince Edward counties as part of the CASTNET (Clean Air Status and Trends Network) program.  Data from these sites are available for download at the Castnet website

Ozone Monitoring Network

National Ambient Air Quality Standards (NAAQS)

Primary Standard for O3:

- Maximum 8-hour average concentration of 0.070 ppm, effective October 1, 2015, based on 3-year average of the annual fourth highest daily maximum 8-hour averages.

- Secondary Standard for O3:

Same as primary

The standard is attained at a monitoring site when the 3-year average of the fourth highest daily maximum 8-hour average for each of the three most recent years is less than or equal to 0.070 ppm.

| Site | Days Exceeded 0.070 ppm | 2019 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Highest Daily Maximum 8-Hour Avg. |  |  |  |
|  |  | 1st Max. | 2nd Max. | 3rd Max. | 4th Max. |
| (16-B) Wythe Co. | 0 | .061 | .058 | .057 | .057 |
| (26-F) Rockingham Co. | 0 | .058 | .056 | .055 | .055 |
| (28-J) Frederick Co. | 0 | .061 | .057 | .056 | .055 |
| (33-A) Albemarle Co. | 0 | .060 | .058 | .058 | .057 |
| (19-A6) Roanoke Co. | 0 | .061 | .060 | .060 | .059 |
| (21-C) Rockbridge Co. | 0 | .061 | .059 | .058 | .057 |
| (71-H) Chesterfield Co. | 0 | .066 | .059 | .059 | .059 |
| (72-M) Henrico Co. | 0 | .070 | .068 | .064 | .064 |
| (73-E) Hanover Co. | 0 | .067 | .062 | .062 | .061 |
| (75-B) Charles City Co. | 0 | .067 | .065 | .062 | .061 |
| (179-K) Hampton | 0 | .065 | .061 | .061 | .061 |
| (183-E) Suffolk | 0 | .069 | .061 | .060 | .059 |
| (183-F) Suffolk | 0 | .059 | .058 | .058 | .058 |
| (37-B) Fauquier Co. | 0 | .059 | .056 | .055 | .055 |
| (38-I) Loudoun Co. | 0 | .074 | .063 | .062 | .060 |
| (44-A) Stafford Co. | 0 | .068 | .060 | .060 | .059 |
| (45-L) Prince William Co. | 0 | .070 | .063 | .060 | .060 |
| (46-B9) Fairfax Co. | 0 | .072 | .071 | .070 | .070 |
| (47-T) Arlington Co. | 0 | .074 | .069 | .069 | .068 |
| (48-A) Caroline Co. | 0 | .065 | .061 | .061 | .060 |

2017-2019 Fourth-Highest Daily Maximum 8-Hour Ozone Averages

(units parts per million)

| Areas | Monitor Location (County/City) | 2017 | 2017 | 2019 | 3-Year Average (NAAQS = .070 ppm) |
| --- | --- | --- | --- | --- | --- |
| Richmond Maintenance Area | Chesterfield Co. | .060 | .058 | .059 | .059 |
|  | Henrico Co. | .067 | .065 | .064 | .065 |
|  | Hanover Co. | .064 | .066 | .061 | .063 |
|  | Charles City Co. | .061 | .064 | .061 | .062 |
| Hampton Roads Maintenance Area | Hampton City | .064 | .062 | .061 | .062 |
|  | Suffolk City (TCC) | .060 | .063 | .059 | .060 |
|  | Suffolk City (Holland) | .058 | .059 | .058 | .058 |
| Fredericksburg Maintenance Area | Stafford Co. | .058 | .064 | .059 | .060 |
| Northern Virginia Nonattainment Area | Loudoun Co. | .066 | .065 | .060 | .063 |
|  | Prince William Co. | .065 | .065 | .060 | .063 |
|  | Arlington Co. | .070 | .070 | .068 | .069 |
|  | Fairfax Co. (Lee Park) | .068 | .066 | .070 | .068 |
| Shenandoah National Park Maintenance Area | Madison Co. (Big Meadows) | .062 | .062 | .058 | .060 |
| Areas Currently Designated Attainment | Wythe Co. | .060 | .061 | .057 | .059 |
|  | Rockbridge Co. | .057 | .060 | .057 | .058 |
|  | Rockingham Co. | .059 | .059 | .055 | .057 |
|  | Frederick Co. | .061 | .059 | .055 | .058 |
|  | Albemarle Co. | .059 | .062 | .057 | .059 |
|  | Roanoke Co. | .058 | .062 | .059 | .059 |
|  | Fauquier Co. | .056 | .060 | .055 | .057 |
|  | Caroline Co. | .061 | .062 | .060 | .061 |

A 3-year average greater than .070 ppm exceeds the 8-hour NAAQS for ozone.

Lead Monitoring Network

Lead (Pb) is a metal found naturally in the environment as well as in manufactured products.  In the past, emissions from cars and trucks using leaded gasoline were the primary sources of lead in the atmosphere.  Efforts by EPA to remove lead from motor vehicle gasoline resulted in dramatic reductions of lead in the ambient air from 1980 to 1999.  Now the major sources of lead in the air are ore, metals processing, and piston-engine aircraft operating on leaded aviation gasoline.

Particles containing lead can be inhaled, or lead can be ingested from drinking water or through contaminated food as a result of deposition of leaded particles onto the ground or in the water.  In the body, lead can accumulate in the bones; affect the nervous system, kidney function, immune system, reproductive and developmental systems and the cardiovascular system.  Young children are particularly vulnerable to the effects of lead, where it can contribute to behavioral problems, learning deficits and lowered IQ.  Lead can stay in the environment for a long time, causing adverse effects to plants and animals.

The National Ambient Air Quality Standards, or NAAQS, for lead were revised in October 2008.  At that time, EPA reduced the level of the standard from 1.5 micrograms per cubic meter to 0.15 micrograms per cubic meter.  The secondary standard was also reduced to the level of the new primary standard.  Virginia DEQ received a waiver from EPA in 1997 to discontinue lead monitoring because Virginia had no major lead sources.  However, when the new standards were promulgated, the emission threshold that agencies were required to use for determining if a lead monitor was needed near a source also changed.  As a result, Virginia had to resume monitoring for lead in a few areas, and AQM began installing the lead monitors in late 2009 and completed installation in October 2010.  For additional information on the revised lead standards, see EPA's Lead Standards .

To measure lead, ambient air is drawn into a high volume sampler.  The sample air flows across an 8 x 10 inch glass fiber filter at a rate of 39-60 cubic feet per minute for a 24-hour period.  The filter is sent to the Division of Consolidated Laboratories, where a small portion of it is analyzed using inductively coupled plasma – mass spectrometry (ICP-MS).  The resulting lead concentration is reported as micrograms per cubic meter (g/m3).  The normal sampling schedule is once every sixth day from midnight to midnight.  The lead sampling schedule for 2019 can be found here at Lead Sampling Calendar.

Lead Monitoring Network

National Ambient Air Quality Standards (NAAQS)

Primary Standard for Pb:

- 0.15 μg/m3 three-month rolling average

Secondary Standard for Pb:

- Same as Primary

2019 Pb 3-Month Averages (units in g/m3, LC )

| Site | No. Valid 3-Month Observations | 1st Max | 2nd Max | >0.15 μg/m3 |
| --- | --- | --- | --- | --- |
| (18-C) Montgomery Co. | 12 | 0.00 | 0.00 | 0 |
| (109-N) Roanoke | 12 | 0.01 | 0.01 | 0 |

Acid Precipitation Network

Air Toxics Monitoring Network

Acid Precipitation Network

The National Acid Deposition Program (NADP) had five monitoring sites in Virginia in 2019:  Big Meadows (Shenandoah National Park), Hortons Station (Giles County), Charlottesville, Prince Edward County, and Natural Bridge Station (Rockbridge County).  NADP site information and data are available on-line at National Atmospheric Deposition Program in the NTN (National Trends Network) section.

In addition to the five acid deposition monitors, there was one NADP Mercury Deposition Network (MDN) site in Virginia: Big Meadows (Shenandoah National Park).  MDN site information and data are available on-line at Mercury Deposition Network.

Air Toxics Monitoring Network

In 2019, the Office of Air Quality Monitoring (AQM) of the Department of Environmental Quality (DEQ) operated an Air Toxics Monitoring Network (ATMN).  The ATMN consists of two separate monitoring programs.  The Urban Air Toxics Monitoring Program (UATM), and the National Air Toxics Trend Stations Program (NATTS).

The UATM program consisted of two sites that were located at the Carter G. Woodson Middle School in Hopewell and the DEQ Tidewater Regional Office (TRO) in Virginia Beach.  Sampling at these sites consisted of Volatile Organic Compounds (VOC), Carbonyls, and Total Suspended Particulate (TSP) Metals.  Each of the UATM sites had a sampling schedule consisting of 24-hour samples collected every 6th day.  Data from these sites will be used to characterize air toxics concentrations in the respective urban areas.

AQM used the manual method for collecting ambient air samples for VOC analysis.  Whole air samples were collected using evacuated SilcoT or SUMMAT canisters and RMESI (RM Environmental Systems, Inc.) air samplers.  The Division of Consolidated Laboratory Services (DCLS), the Virginia state laboratory, analyzed samples. DCLS used a Gas Chromatograph equipped with a Mass Selective Detector and employed method TO15.  Carbonyls were collected on DNPH (2,4-Dinitrophenylhydrazine) treated sorbent tubes using ATEC 8000 cartridge samplers.  Samples taken were analyzed by the DCLS, using a Liquid Chromatographic procedure designated as method TO11A.  Metals samples were collected using a high volume Total Suspended Particulate (TSP) sampler and were analyzed by the DCLS.  Analysis utilized inductively coupled plasma mass spectrometry (ICP-MS) using method IO-3.1 and IO-3.5.

The NATTS program operated one station located at the MathScience Innovation Center (MSIC) in Henrico County.  The NATTS site had a sampling schedule consisting of 24-hour samples collected every 6th day.  Data from this site will be evaluated along with data from all of the NATTS sites nationally.  AQM used the manual method for collecting ambient air samples for VOC analysis.  Whole air samples were collected using evacuated SilcoT canisters and RMESI (RM Environmental Systems, Inc.) air samplers.  Each sample was analyzed by the DCLS, using a Gas Chromatograph equipped with a Mass Selective Detector, utilizing method TO15.  Carbonyls were collected on DNPH (2,4-Dinitrophenylhydrazine) treated sorbent tubes using ATEC 8000 cartridge samplers.  Samples were analyzed by DCLS using a Liquid Chromatographic procedure, and the TO11A method.  The Metals samples were collected using a high volume 10-micron Particulate Matter (PM10) sampler and analyzed by the DCLS.  Analysis utilized Inductively Coupled Plasma Mass Spectrometry (ICP-MS) using method IO-3.1 and IO-3.5.

Air Toxics Monitoring Network

Detectable VOC in 24-Hour Canister Samples

GC/MSD - MathScience Innovation Center (NATTS Site), Henrico County, VA

January 1 to December 31, 2019- Concentrations are in ppbV

(non detects are counted as zeros for statistical purposes)

| Parameter | Compound Name | Num | Minimum | Maximum | Median | Average | StDev |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 43205 | Propylene | 60 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 43207 | Freon 113 | 60 | 0.06 | 0.12 | 0.08 | 0.076 | 0.012 |
| 43208 | Freon 114 | 60 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 43209 | Ethyl Acetate | 60 | 0.00 | 0.23 | 0.00 | 0.036 | 0.063 |
| 43218 | 1,3-butadiene | 60 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 43231 | Hexane | 60 | 0.00 | 0.61 | 0.10 | 0.114 | 0.108 |
| 43232 | Heptane | 60 | 0.00 | 0.24 | 0.00 | 0.033 | 0.050 |
| 43248 | Cyclohexane | 60 | 0.00 | 0.06 | 0.00 | 0.003 | 0.011 |
| 43372 | MTBE | 60 | 0.00 | 0.06 | 0.00 | 0.001 | 0.008 |
| 43441 | Methyl Methacrylate | 60 | 0.00 | 0.07 | 0.00 | 0.002 | 0.012 |
| 43505 | Acrolein | 60 | 0.00 | 2.11 | 0.22 | 0.351 | 0.469 |
| 43702 | Acetonitrile | 60 | 0.00 | 25.90 | 0.59 | 1.661 | 3.627 |
| 43704 | Acrylonitrile | 60 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 43801 | Chloromethane | 60 | 0.00 | 0.80 | 0.58 | 0.563 | 0.107 |
| 43802 | Dichloromethane | 60 | 0.07 | 1.00 | 0.23 | 0.306 | 0.246 |
| 43803 | Chloroform | 60 | 0.01 | 0.08 | 0.02 | 0.025 | 0.010 |
| 43804 | Carbon Tetrachloride | 60 | 0.04 | 0.11 | 0.08 | 0.074 | 0.013 |
| 43806 | Bromoform | 60 | 0.00 | 0.05 | 0.00 | 0.001 | 0.007 |
| 43811 | Trichlorofluoromethane | 60 | 0.20 | 0.35 | 0.26 | 0.259 | 0.039 |
| 43812 | Chloroethane | 60 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 43813 | 1,1-Dichloroethane | 60 | 0.00 | 0.06 | 0.00 | 0.002 | 0.010 |
| 43814 | Methyl chloroform | 60 | 0.00 | 0.06 | 0.00 | 0.002 | 0.010 |
| 43815 | Ethylene dichloride | 60 | 0.00 | 0.08 | 0.00 | 0.002 | 0.012 |
| 43817 | Tetrachloroethylene | 60 | 0.00 | 0.08 | 0.02 | 0.018 | 0.019 |
| 43818 | 1,1,2,2-Tetrachloroethane | 60 | 0.00 | 0.05 | 0.00 | 0.002 | 0.008 |
| 43819 | Bromomethane | 60 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 |
| 43820 | 1,1,2-Trichloroethane | 60 | 0.00 | 0.06 | 0.00 | 0.002 | 0.010 |
| 43823 | Dichlorodifluoromethane | 60 | 0.38 | 0.57 | 0.49 | 0.487 | 0.044 |
| 43824 | Trichloroethylene | 60 | 0.00 | 0.06 | 0.00 | 0.002 | 0.010 |
| 43826 | 1,1-Dichloroethylene | 60 | 0.00 | 0.06 | 0.00 | 0.002 | 0.009 |
| 43828 | Bromodichloromethane | 60 | 0.00 | 0.04 | 0.00 | 0.001 | 0.007 |
| 43829 | 1,2-Dichloropropane | 60 | 0.00 | 0.07 | 0.00 | 0.002 | 0.011 |
| 43830 | trans-1,3-Dichlopropylene | 60 | 0.00 | 0.03 | 0.00 | 0.001 | 0.004 |
| 43831 | cis-1,3-Dichlopropylene | 60 | 0.00 | 0.03 | 0.00 | 0.001 | 0.005 |
| 43832 | Dibromochloromethane | 60 | 0.00 | 0.03 | 0.00 | 0.001 | 0.005 |
| 43838 | Trans-1,2-Dichloroethene | 60 | 0.00 | 0.06 | 0.00 | 0.002 | 0.009 |
| 43839 | cis-1,2-Dichloroethene | 60 | 0.00 | 0.06 | 0.00 | 0.002 | 0.010 |
| 43843 | Ethylene Dibromide | 60 | 0.00 | 0.08 | 0.00 | 0.004 | 0.013 |
| 43844 | Hexachlorobutadiene | 60 | 0.00 | 0.06 | 0.00 | 0.003 | 0.010 |
| 43860 | Vinyl Chloride | 60 | 0.00 | 0.06 | 0.00 | 0.001 | 0.008 |
| 45109 | m/p-Xylene | 60 | 0.00 | 0.46 | 0.07 | 0.084 | 0.073 |
| 45201 | Benzene | 60 | 0.07 | 0.92 | 0.14 | 0.167 | 0.125 |
| 45202 | Toluene | 60 | 0.05 | 1.28 | 0.21 | 0.240 | 0.192 |
| 45203 | Ethylbenzene | 60 | 0.00 | 0.17 | 0.03 | 0.031 | 0.029 |
| 45204 | o-Xylene | 60 | 0.00 | 0.18 | 0.03 | 0.036 | 0.030 |
| 45207 | 1,3,5-Trimethylbenzene | 60 | 0.00 | 0.05 | 0.00 | 0.008 | 0.014 |
| 45208 | 1,2,4-Trimethylbenzene | 60 | 0.00 | 0.24 | 0.06 | 0.087 | 0.074 |
| 45213 | p-Ethyltoluene | 60 | 0.00 | 0.10 | 0.00 | 0.013 | 0.024 |
| 45220 | Styrene | 60 | 0.00 | 0.30 | 0.00 | 0.034 | 0.059 |
| 45801 | Chlorobenzene | 60 | 0.00 | 0.13 | 0.00 | 0.006 | 0.020 |
| 45805 | 1,2-Dichlorobenzene | 60 | 0.00 | 0.38 | 0.00 | 0.021 | 0.055 |
| 45806 | 1,3-Dichlorobenzene | 60 | 0.00 | 0.53 | 0.00 | 0.039 | 0.091 |
| 45807 | 1,4-Dichlorobenzene | 60 | 0.00 | 0.78 | 0.00 | 0.036 | 0.107 |
| 45810 | 1,2,4-Trichlorobenzene | 60 | 0.00 | 1.78 | 0.00 | 0.081 | 0.257 |
| 46401 | Tetrahydrofuran | 60 | 0.00 | 0.29 | 0.00 | 0.033 | 0.060 |

Detectable VOC in 24-Hour Canister Samples

GC/MSD - Carter G. Woodson Middle School (UATM Site), Hopewell, VA

January 1 to December 31, 2019 - Concentrations are in ppbV

(NonDetects are considered zeros for statistical purposes.  Results below MDLs and/or Reporting Limits are reported.)

| Parameter | Compound Name | Num | Minimum | Maximum | Median | Average | StDev |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 43205 | Propylene | 59 | 0.00 | 0.52 | 0.00 | 0.025 | 0.096 |
| 43207 | 1,1,2-Trichlorotrifluoroethane | 59 | 0.06 | 0.16 | 0.08 | 0.082 | 0.020 |
| 43208 | Freon 114 | 59 | 0.00 | 0.12 | 0.00 | 0.002 | 0.016 |
| 43209 | Ethyl Acetate | 59 | 0.00 | 0.28 | 0.00 | 0.016 | 0.047 |
| 43218 | 1,3-Butadiene | 59 | 0.00 | 0.12 | 0.00 | 0.002 | 0.016 |
| 43231 | Hexane | 59 | 0.00 | 0.46 | 0.08 | 0.095 | 0.101 |
| 43232 | Heptane | 59 | 0.00 | 0.56 | 0.05 | 0.064 | 0.099 |
| 43248 | Cyclohexane | 59 | 0.00 | 0.10 | 0.00 | 0.002 | 0.014 |
| 43372 | MTBE | 59 | 0.00 | 0.09 | 0.00 | 0.002 | 0.012 |
| 43441 | Methyl Methacrylate | 59 | 0.00 | 0.08 | 0.00 | 0.001 | 0.010 |
| 43505 | Acrolein | 59 | 0.00 | 7.12 | 0.27 | 0.549 | 1.045 |
| 43702 | Acetonitrile | 59 | 0.00 | 294.91 | 0.00 | 5.678 | 38.549 |
| 43704 | Acrylonitrile | 59 | 0.00 | 0.09 | 0.00 | 0.002 | 0.012 |
| 43801 | Chloromethane | 59 | 0.43 | 0.76 | 0.58 | 0.574 | 0.074 |
| 43802 | Methylene Chloride | 59 | 0.06 | 0.35 | 0.09 | 0.106 | 0.051 |
| 43803 | Trichloromethane | 59 | 0.00 | 0.11 | 0.02 | 0.025 | 0.014 |
| 43804 | Carbon Tetrachloride | 59 | 0.00 | 0.14 | 0.07 | 0.065 | 0.019 |
| 43806 | Bromoform | 59 | 0.00 | 0.05 | 0.00 | 0.002 | 0.008 |
| 43811 | Freon 11 | 59 | 0.00 | 0.32 | 0.24 | 0.240 | 0.042 |
| 43812 | Ethyl Chloride | 59 | 0.00 | 0.13 | 0.00 | 0.002 | 0.017 |
| 43813 | 1,1-Dichloroethane | 59 | 0.00 | 0.09 | 0.00 | 0.002 | 0.012 |
| 43814 | 1,1,1-Trichloroethane | 59 | 0.00 | 0.09 | 0.00 | 0.002 | 0.012 |
| 43815 | 1,2-Dichloroethane | 59 | 0.00 | 0.10 | 0.00 | 0.002 | 0.013 |
| 43817 | Tetrachloroethylene | 59 | 0.00 | 0.11 | 0.00 | 0.009 | 0.019 |
| 43818 | 1,1,2,2-Tetrachloroethane | 59 | 0.00 | 0.08 | 0.00 | 0.003 | 0.012 |
| 43819 | Bromomethane | 59 | 0.00 | 0.12 | 0.00 | 0.002 | 0.016 |
| 43820 | 1,1,2-Trichloroethane | 59 | 0.00 | 0.09 | 0.00 | 0.004 | 0.014 |
| 43823 | Dichlorodifluoromethane | 59 | 0.39 | 0.57 | 0.48 | 0.480 | 0.041 |
| 43824 | Trichloroethylene | 59 | 0.00 | 0.11 | 0.00 | 0.004 | 0.015 |
| 43826 | 1,1-Dichloroethene | 59 | 0.00 | 0.10 | 0.00 | 0.002 | 0.013 |
| 43828 | Bromodichloromethane | 59 | 0.00 | 0.10 | 0.00 | 0.002 | 0.013 |
| 43829 | 1,2-Dichloropropane | 59 | 0.00 | 0.13 | 0.00 | 0.002 | 0.017 |
| 43830 | trans-1,3-Dichloro-1-propene | 59 | 0.00 | 0.07 | 0.00 | 0.001 | 0.009 |
| 43831 | cis-1,3-Dichloropropene | 59 | 0.00 | 0.08 | 0.00 | 0.001 | 0.010 |
| 43832 | Dibromochloromethane | 59 | 0.00 | 0.07 | 0.00 | 0.002 | 0.010 |
| 43838 | trans-1,2-Dichloroethene | 59 | 0.00 | 0.09 | 0.00 | 0.002 | 0.012 |
| 43839 | cis-1,2-Dichloroethene | 59 | 0.00 | 0.08 | 0.00 | 0.001 | 0.010 |
| 43843 | 1,2-Dibromoethane | 59 | 0.00 | 0.08 | 0.00 | 0.006 | 0.018 |
| 43844 | Hexachloro-1,3-Butadiene | 59 | 0.00 | 0.08 | 0.00 | 0.003 | 0.012 |
| 43860 | Vinyl Chloride | 59 | 0.00 | 0.10 | 0.00 | 0.002 | 0.013 |
| 45109 | m,p-Xylenes | 59 | 0.00 | 0.36 | 0.06 | 0.077 | 0.068 |
| 45201 | Benzene | 59 | 0.06 | 0.63 | 0.13 | 0.151 | 0.096 |
| 45202 | Toluene | 59 | 0.03 | 0.97 | 0.15 | 0.184 | 0.156 |
| 45203 | Ethylbenzene | 59 | 0.00 | 0.11 | 0.02 | 0.028 | 0.029 |
| 45204 | o-Xylene | 59 | 0.00 | 0.15 | 0.03 | 0.033 | 0.030 |
| 45207 | 1,3,5-Trimethylbenzene | 59 | 0.00 | 0.05 | 0.00 | 0.010 | 0.015 |
| 45208 | 1,2,4-Trimethylbenzene | 59 | 0.00 | 0.24 | 0.06 | 0.085 | 0.075 |
| 45213 | 4-Ethyltoluene | 59 | 0.00 | 0.22 | 0.00 | 0.024 | 0.046 |
| 45220 | Styrene | 59 | 0.00 | 0.54 | 0.00 | 0.063 | 0.106 |
| 45801 | Chlorobenzene | 59 | 0.00 | 0.13 | 0.00 | 0.012 | 0.030 |
| 45805 | 1,2-Dichlorobenzene | 59 | 0.00 | 0.30 | 0.00 | 0.035 | 0.075 |
| 45806 | 1,3-Dichlorobenzene | 59 | 0.00 | 0.41 | 0.00 | 0.052 | 0.103 |
| 45807 | 1,4-Dichlorobenzene | 59 | 0.00 | 0.51 | 0.00 | 0.055 | 0.118 |
| 45810 | 1,2,4-Trichlorobenzene | 59 | 0.00 | 1.03 | 0.00 | 0.106 | 0.239 |
| 46401 | Tetrahydrofuran | 59 | 0.00 | 0.22 | 0.00 | 0.009 | 0.033 |

24 Hour Carbonyl Sampling 2019 Summary Statistical Analysis

Concentrations are in ug/m3

(non detects are counted as zeros for statistical purposes)

| Site | Parameter | Compound Name | Num | Minimum | Maximum | Median | Average | StDev |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carter G. Woodson Middle School | 43502 | Formaldehyde | 60 | 0.47 | 8.78 | 1.88 | 2.585 | 1.992 |
|  | 43503 | Acetaldehyde | 60 | 0.69 | 3.04 | 1.51 | 1.575 | 0.546 |
|  | 43504 | Propionaldehyde | 60 | 0.10 | 0.63 | 0.23 | 0.260 | 0.123 |
|  | 43552 | Methyl Ethyl Ethyl Ketone | 60 | 0.17 | 0.84 | 0.40 | 0.411 | 0.137 |
|  | 43560 | Methyl Isobutyl Ketone | 59 | 0.00 | 0.24 | 0.00 | 0.013 | 0.045 |

NATTS Carbonyl Sampling 2019 Summary Statistical Analysis

Concentrations are in ug/m3

(non detects are counted as zeros for statistical purposes)

| Site | Parameter | Compound Name | Num | Minimum | Maximum | Median | Average | StDev |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MathScience Innovation Center | 43502 | Formaldehyde | 57 | 0.73 | 8.94 | 2.64 | 3.260 | 1.980 |
|  | 43503 | Acetaldehyde | 57 | 0.67 | 2.66 | 1.28 | 1.370 | 0.490 |
|  | 43504 | Propionaldehyde | 57 | 0.12 | 0.70 | 0.25 | 0.290 | 0.150 |
|  | 43551 | Acetone | 57 | 0.45 | 8.50 | 2.85 | 2.990 | 1.520 |
|  | 43552 | Methyl Ethyl Ketone | 57 | 0.14 | 1.05 | 0.42 | 0.440 | 0.190 |
|  | 43560 | Methyl Isobutyl Ketone | 57 | 0.00 | 0.17 | 0.00 | 0.029 | 0.047 |

TSP Metals Sampling 2019 Summary Statistical Analysis

Concentrations are in ng/m3

(non detects are counted as zeros for statistical purposes)

| Site | Parameter | Compound Name | Num | Minimum | Maximum | Median | Average | StDev |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carter G. Woodson Middle School | 12103 | Arsenic | 61 | 0.39 | 13.00 | 1.24 | 1.822 | 1.934 |
|  | 12105 | Beryllium | 61 | 0.00 | 0.03 | 0.00 | 0.006 | 0.009 |
|  | 12110 | Cadmium | 61 | 0.00 | 0.55 | 0.10 | 0.126 | 0.117 |
|  | 12112 | Chromium | 61 | 1.45 | 6.19 | 2.58 | 2.801 | 0.888 |
|  | 12128 | Lead | 61 | 1.34 | 30.79 | 3.26 | 4.656 | 4.790 |
|  | 12132 | Manganese | 61 | 3.28 | 43.54 | 11.40 | 14.494 | 9.040 |
|  | 12136 | Nickel | 61 | 0.76 | 4.57 | 1.34 | 1.467 | 0.547 |

| Site | Parameter | Compound Name | Num | Minimum | Maximum | Median | Average | StDev |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tidewater Regional Office | 12103 | Arsenic | 59 | 0.24 | 5.52 | 0.70 | 1.055 | 0.971 |
|  | 12105 | Beryllium | 59 | 0.00 | 0.01 | 0.00 | 0.002 | 0.004 |
|  | 12110 | Cadmium | 59 | 0.00 | 0.24 | 0.04 | 0.062 | 0.059 |
|  | 12112 | Chromium | 59 | 1.06 | 2.94 | 1.75 | 1.818 | 0.552 |
|  | 12128 | Lead | 59 | 0.52 | 15.84 | 1.82 | 2.276 | 2.145 |
|  | 12132 | Manganese | 59 | 1.11 | 21.07 | 5.64 | 6.184 | 3.871 |
|  | 12136 | Nickel | 59 | 0.47 | 1.65 | 0.87 | 0.947 | 0.293 |

NATTS PM10 Metals Sampling 2019 Summary Statistical Analysis

Concentrations are in ng/m3

(non-detects and negative values are counted as zeros for statistical purposes)

| Site | Parameter | Compound Name | Num | Minimum | Maximum | Median | Average | StDev |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MathScience Innovation Center | 85103 | Arsenic | 59 | 0.10 | 8.71 | 0.58 | 0.833 | 1.154 |
|  | 85105 | Berrylium | 59 | 0.00 | 0.02 | 0.00 | 0.003 | 0.006 |
|  | 85110 | Cadmium | 59 | 0.00 | 0.32 | 0.05 | 0.069 | 0.071 |
|  | 85112 | Chromium | 59 | 0.91 | 2.15 | 1.28 | 1.303 | 0.225 |
|  | 85128 | Lead | 59 | 0.39 | 8.41 | 1.45 | 1.858 | 1.137 |
|  | 85132 | Manganese | 59 | 0.45 | 6.72 | 2.32 | 2.297 | 1.119 |
|  | 85136 | Nickel | 59 | 0.40 | 0.98 | 0.68 | 0.682 | 0.118 |

AQI (Air Quality Index)

What is the AQI?

The air quality index (AQI) is a measurement designed to indicate how clean the air is in an area, and it also provides information about health effects associated with air pollution.  The index is reported daily, or in some cases continuously, and calculated from measured concentrations of five major pollutants regulated by the Clean Air Act:  ground-level ozone, particulate matter, carbon monoxide, sulfur dioxide, and nitrogen dioxide.  EPA has established national ambient air quality standards (NAAQS) for each of these pollutants to protect public health, and the index is derived from the NAAQS.  State and local agencies are required to report the AQI in areas where the population is 350,000 or more, although it is often reported in additional areas as a public service.

How does the AQI work?

The AQI range is from 0 to 500, with the low numbers representing good air quality and the high numbers indicating unhealthy or even hazardous air quality.  The index is divided into six categories with coordinating color codes.  In addition, each category has a health-related message associated with it, to inform the public of possible health effects that may arise as a result of breathing polluted air.

Generally, an index of 100 corresponds to the national air quality standard for the pollutant, which is the level that EPA has established to protect public health.  Levels below 100 are considered satisfactory, while numbers above 100 are considered unhealthy, first for sensitive groups, and then for the general public as the index value increases.

How is the AQI calculated?

The AQI is calculated from air pollution measurements collected at monitoring sites across the country.  The reporting agency must calculate an index for each pollutant from the measured concentrations at all monitoring sites in an area using a standard formula developed by EPA.  The pollutant with the highest index is reported as the “primary pollutant”, and the highest index is reported as the AQI for the area.  If the AQI is above 100, then the agency must report which groups may be sensitive to the primary pollutant.  If two or more pollutants have indexes above 100, then the agency must report all groups that may be affected by those pollutants.

How do I find the AQI for my area?

DEQ reports the air quality index for Roanoke, Winchester, Richmond, Hampton Roads, and Northern Virginia on its Air Quality Index webpage.  Air quality forecasts and current air quality data can be obtained at the DEQ site, as well as links to other air quality websites.  EPA also reports air quality conditions for the United States at AirNow.

In addition to the internet, current and forecasted AQI levels are broadcast on local television and radio weather reports in many areas, as well as printed in newspapers.  By reaching out to the public using these different media, individuals can plan their activities to reduce exposure during episodes of poor air quality, and they can also take steps to reduce pollution.

For detailed information about the AQI, and on health effects of the pollutants that are included in the AQI, visit AirNow.

| Air Quality Index (0-500) | Levels of Health Concern | Meaning |
| --- | --- | --- |
| (0-50) | Good | Air quality is considered satisfactory, and air pollution poses little or no risk. |
| (51-100) | Moderate | Air quality is acceptable; however, for some pollutants there may be a moderate health concern for a very small number of people who are unusually sensitive to air pollution. |
| (101-150) | Unhealthy for Sensitive Groups | Members of sensitive groups may experience health effects. The general public is not likely to be affected. |
| (151-200) | Unhealthy | Everyone may begin to experience health effects; members of sensitive groups may experience more serious health effects. |
| (201-300) | Very Unhealthy | Health warnings of emergency conditions. The entire population is more likely to be affected. |
| (301-500) | Hazardous | Health alert: everyone may experience more serious health effects. |

Note: Values above 500 are considered Beyond the AQI. Follow recommendations for the "Hazardous category." Additional information on reducing exposure to extremely high levels of particle pollution is available “here”

OZONE

### What is ozone?

Ozone is a gas that occurs both in the Earth's upper atmosphere and at ground level. Ozone can be "good" or "bad" for your health and the environment, depending on its location in the atmosphere.

### How Can Ozone Be Both Good and Bad?

Ozone occurs in two layers of the atmosphere. The layer closest to the Earth's surface is the troposphere. Here, ground-level or "bad" ozone is an air pollutant that is harmful to breathe and it damages crops, trees and other vegetation. It is a main ingredient of urban smog. The troposphere generally extends to a level about 6 miles up, where it meets the second layer, the stratosphere. The stratosphere or "good" ozone layer extends upward from about 6 to 30 miles and protects life on Earth from the sun's harmful ultraviolet (UV) rays.

### What is Happening to the "Good" Ozone Layer?

Ozone is produced naturally in the stratosphere. But this "good" ozone is gradually being destroyed by man-made chemicals referred to as ozone-depleting substances (ODS), including chlorofluorocarbons (CFCs), hydrochlorofluorocarbons (HCFCs), halons, methyl bromide, carbon tetrachloride, and methyl chloroform. These substances were formerly used and sometimes still are used in coolants, foaming agents, fire extinguishers, solvents, pesticides, and aerosol propellants. Once released into the air these ozone-depleting substances degrade very slowly. In fact, they can remain intact for years as they move through the troposphere until they reach the stratosphere. There they are broken down by the intensity of the sun's UV rays and release chlorine and bromine molecules, which destroy the "good" ozone. Scientists estimate that one chlorine atom can destroy 100,000 "good" ozone molecules.

Even though we have reduced or eliminated the use of many ODSs, their use in the past can still affect the protective ozone layer. Research indicates that depletion of the "good" ozone layer is being reduced worldwide. Thinning of the protective ozone layer can be observed using satellite measurements, particularly over the Polar Regions.

### How Does the Depletion of "Good" Ozone Affect Human Health and the Environment?

Ozone depletion can cause increased amounts of UV radiation to reach the Earth which can lead to more cases of skin cancer, cataracts, and impaired immune systems. Overexposure to UV is believed to be contributing to the increase in melanoma, the most fatal of all skin cancers. Since 1990, the risk of developing melanoma has more than doubled.

UV can also damage sensitive crops, such as soybeans, and reduce crop yields. Some scientists suggest that marine phytoplankton, which are the base of the ocean food chain, are already under stress from UV radiation. This stress could have adverse consequences for human food supplies from the oceans.

### What is Being Done About the Depletion of "Good" Ozone?

The United States, along with over 180 other countries, recognized the threats posed by ozone depletion and in 1987 adopted a treaty called the Montreal Protocol to phase out the production and use of ozone-depleting substances.

EPA has established regulations to phase out ozone-depleting chemicals in the United States. Warning labels must be placed on all products containing CFCs or similar substances and nonessential uses of ozone-depleting products are prohibited. Releases into the air of refrigerants used in car and home air conditioning units and appliances are also prohibited. Some substitutes to ozone-depleting products have been produced and others are being developed. If the United States and other countries stop producing ozone-depleting substances, natural ozone production should return the ozone layer to normal levels by about 2050.

### What Causes "Bad" Ozone?

Ground-level or "bad" ozone is not emitted directly into the air, but is created by chemical reactions between oxides of nitrogen (NOx) and volatile organic compounds (VOC) in the presence of sunlight. Emissions from industrial facilities and electric utilities, motor vehicle exhaust, gasoline vapors, and chemical solvents are some of the major sources of NOx and VOC.

At ground level, ozone is a harmful pollutant. Ozone pollution is a concern during the summer months because strong sunlight and hot weather result in harmful ozone concentrations in the air we breathe. Many urban and suburban areas throughout the United States have high levels of "bad" ozone. But many rural areas of the country are also subject to high ozone levels as winds carry emissions hundreds of miles away from their original sources.

### How Does "Bad" Ozone Affect Human Health and the Environment?

Breathing ozone can trigger a variety of health problems including chest pain, coughing, and throat irritation. It can worsen bronchitis, emphysema, and asthma. "Bad" ozone also can reduce lung function and inflame the linings of the lungs. Repeated exposure may permanently scar lung tissue.

Healthy people also experience difficulty breathing when exposed to ozone pollution. Because ozone forms in hot weather, anyone who spends time outdoors in the summer may be affected, particularly children, older people, outdoor workers and people exercising. Millions of Americans live in areas where the national ozone health standards are exceeded.

Ground-level or "bad" ozone also damages vegetation and ecosystems. It leads to reduced agricultural crop and commercial forest yields, reduced growth and survivability of tree seedlings, and increased susceptibility to diseases, pests and other stresses such as harsh weather. In the United States alone, ground-level ozone is responsible for an estimated $500 million in reduced crop production each year. Ground-level ozone also damages the foliage of trees and other plants, affecting the landscape of cities, national parks and forests, and recreation areas.

### What Is Being Done About "Bad" Ozone?

Under the Clean Air Act, EPA has set protective health-based standards for ozone in the air we breathe. EPA, state, and cities have instituted a variety of multi-faceted programs to meet these health-based standards. Throughout the country, additional programs are being put into place to cut NOx and VOC emissions from vehicles, industrial facilities, and electric utilities. Programs are also aimed at reducing pollution by reformulating fuels and consumer/commercial products, such as paints and chemical solvents, that contain VOC. Voluntary programs also encourage communities to adopt practices, such as carpooling, to reduce harmful emissions.

High-Altitude "Good" Ozone

- Protect yourself against sunburn. When the UV Index is "high" or "very high": Limit outdoor activities between 10 am and 4 pm, when the sun is most intense. Twenty minutes before going outside, liberally apply a broad-spectrum sunscreen with a Sun Protection Factor (SPF) of at least 15. Reapply every two hours or after swimming or sweating. For UV Index forecasts, check local media reports or visit: sun safety

- Use approved refrigerants in air conditioning and refrigeration equipment. Make sure technicians that work on your car or home air conditioners or refrigerator are certified to recover the refrigerant. Repair leaky air conditioning units before refilling them.

### Ground-Level "Bad" Ozone

- Check the air quality forecast in your area. At times when the Air Quality Index (AQI) is forecast to be unhealthy, limit physical exertion outdoors. In many places, ozone peaks in mid-afternoon to early evening. Change the time of day of strenuous outdoor activity to avoid these hours, or reduce the intensity of the activity. For AQI forecasts, check your local media reports or visit: http://epa.gov/airnow

- Help your local electric utilities reduce ozone air pollution by conserving energy at home and the office. Consider setting your thermostat a little higher in the summer. Participate in your local utilities' load-sharing and energy conservation programs.

- Reduce air pollution from cars, trucks, gas-powered lawn and garden equipment, boats and other engines by keeping equipment properly tuned and maintained. During the summer, fill your gas tank during the cooler evening hours and be careful not to spill gasoline. Reduce driving, carpool, use public transportation, walk, or bicycle to reduce ozone pollution, especially on hot summer days.

- Use household and garden chemicals wisely. Use low VOC paints and solvents. And be sure to read labels for proper use and disposal.

For more information, please visit these sites:

Air Topics

Airnow

A number of factors influence ozone formation, including emissions from cars, trucks,

buses, power plants, and industries, along with weather conditions.  Weather is especially favorable for ozone formation when it’s hot, dry and sunny, and winds are calm and light. Federal and state regulations, including regulations for power plants, vehicles and fuels, are helping reduce ozone pollution nationwide.

For the 35 metropolitan areas in this story map, the graphic below shows the total number of unhealthy days for ozone pollution since the year 2000.

For more information, please visit these EPA sites:

A Look Back in 2019

National Air Quality: Status and Trends of Key Air Pollutants

Appendix A

Abbreviation Table

AQM				Air Quality Monitoring

AQCR				Air Quality Control Region

ATMN				Air Toxics Monitoring Network

Avg.				Average

CAMP				Community Air Toxics Assessment Monitoring Program

CASTNET			Clean Air Status and Trends Network

CO				Carbon Monoxide

DEQ				Department of Environmental Quality

FEM				Federal Equivalent Method

EPA				Environmental Protection Agency

FRM				Federal Reference Method

IMPROVE			Interagency Monitoring of Protected Visual Environments

LAT				Latitude

LC				Local Conditions

LONG				Longitude

MARAMA			Mid-Atlantic Regional Air Management Association

MET.				Meteorological Instrumentation

MSA				Metropolitan Statistical Area

NA				Not Available

NADP				National Acid Deposition Program

NAMS				National Air Monitoring Stations

NATTS				National Air Toxics Trend Stations

NMOC				Non-Methane Organic Compounds

NO2				Nitrogen Dioxide

NUM				Number of Samples

O3				Ozone

PAMHC				Total PAMS Hydrocarbon

PAMS				Photochemical Assessment Monitoring Station

PM10	Particulate Matter with an aerodynamic diameter less than or equal to 10 microns

PM2.5				Particulate Matter with an aerodynamic diameter less than or

equal to 2.5 microns

POLLUT.			Pollutant

ppbC				Part Per Billion of Carbon

ppbv				Part Per Billion by volume

ppm				Part Per Million

SESARM 			Southeastern States Air Resource Managers, Inc.

SLAMS				State and Local Air Monitoring Station

SO2				Sulfur Dioxide

STD				Standard

STDEV				Standard Deviation

TEOM				Tapered Element Oscillating Microbalance (a method for

continuously measuring PM2.5 in ambient air)

TNMOC			Total Nonmethane Organic Compound

UATM				Urban Air Toxics Monitoring Program

µg/m3				Micrograms per cubic meter

VOC				Volatile Organic Compounds

Southwest Monitoring Network

| STATION NUMBER | POLLUT. | LOCATION | EPA ID | CITY/COUNTY | LAT/LONG |
| --- | --- | --- | --- | --- | --- |
| 16-B | O3 | Sewage Disposal Plant | 51-197-0002 | Rural Retreat Wythe Co. | 36.89117 -81.25423 |
| 23-A | PM10 | Gladeville Elementary School | 51-035-0001 | Galax Carroll Co. | 36.70067 -80.87978 |
| 101-E | PM2.5 | Highland View Elementary School | 51-520-0006 | Bristol | 36.60800 -82.16410 |

Contact Information for this Region:

Southwest Regional Office

Jeffery Hurst, Director

355-A Deadmore Street

Abingdon, VA 24210

(276) 676-4800

Valley Monitoring Network

| STATION NUMBER | POLLUT. | LOCATION | EPA ID | CITY/COUNTY | LAT/LONG |
| --- | --- | --- | --- | --- | --- |
| 21-C | O3, IMPROVE | Natural Bridge Ranger Station | 51-163-0003 | Rockbridge Co. | 37.62668 -79.51257 |
| 26-F | PM2.5, SO2, NO2,O3 | Rockingham VDOT | 51-165-0003 | Harrisonburg Rockingham Co. | 38.47753 -78.81952 |
| 28-J | O3, PM2.5, TEOM | Woodbine Road Lester Building Systems | 51-069-0010 | Rest Frederick Co. | 39.28102 -78.08157 |
| 33-A | O3, PM2.5, PM2.5 cont. | Albemarle High School | 51-003-0001 | Albemarle Co. | 38.07657 -78.50397 |
| 134-C | PM10 | Winchester Courts Building | 51-840-0002 | Winchester | 39.18397 -78.16308 |

Contact information for this Region:

Valley Regional Office

Amy T. Owens, Director

P.O. Box 3000

4411 Early Road

Harrisonburg, VA 22801

(540) 574-7808

Blue Ridge Monitoring Network

| STATION NUMBER | POLLUT. | LOCATION | EPA ID | CITY/COUNTY | LAT/LONG |
| --- | --- | --- | --- | --- | --- |
| 18-C | Lead | Stroubles Creek Wastewater Treatment Plant | 51-121-0009 | Montgomery Co. | 37.18494 -80.51606 |
| 19-A6 | CO,SO2, NO2, O3, PM2.5 | East Vinton Elementary School Ruddell Road | 51-161-1004 | Vinton Roanoke Co. | 37.28342 -79.88452 |
| 109-N | Lead | Mario Industries | 51-770-0016 | Roanoke | 37.27494 -79.98567 |
| 110-C | PM2.5 | Salem High School | 51-775-0011 | Salem | 37.29788 -80.08102 |

| STATION NUMBER | POLLUT. | LOCATION | EPA ID | CITY/COUNTY | LAT/LONG |
| --- | --- | --- | --- | --- | --- |
| 9-I | SO2 | Giles County | 51-071-0007 | Giles County | 37.3863 -80.6539 |
| 20-E | SO2 | Botetourt County | 51-023-0004 | Botetourt County | 37.44796 -79.98649 |
| 104-M | SO2 | Covington City | 51-580-0008 | Covington City | 37.79139 -79.9908 |

Contact information for this Region:

Blue Ridge Regional Office

Robert Weld, Director

901 Russell Drive

Salem, VA 24153

(540) 562-6870

Blue Ridge Monitoring Network

| STATION NUMBER | POLLUT. | LOCATION | EPA ID | CITY/COUNTY | LAT/LONG |
| --- | --- | --- | --- | --- | --- |
| 155-Q | PM2.5 | Leesville Hwy. & Greystone Dr. | 51-680-0015 | Lynchburg | 37.33175 -79.21478 |

Contact information for this Region:

Blue Ridge Regional Office

Robert Weld, Director

901 Russell Drive

Salem, VA 24153

(540) 562-6870

Piedmont Monitoring Network

| STATION NUMBER | POLLUT. | LOCATION | EPA ID | CITY/COUNTY | LAT/LONG |
| --- | --- | --- | --- | --- | --- |
| 71-H | O3 | Beach Road Highway Shop | 51-041-0004 | Chesterfield Co. | 37.35748 -77.59355 |
| 72-M | O3, VOC, PM2.5, PM10,TEOM, Speciation, Toxics, Met, NCore, NATTS, PAMS | MathScience Innovation Center 2401 Hartman Street | 51-087-0014 | Henrico Co. | 37.55652 -77.40027 |
| 72-N | PM2.5 | DEQ-Piedmont Regional Office 4949-A Cox Road | 51-087-0015 | Henrico Co. | 37.67132 -77.56640 |
| 73-E | O3 | McClellan Road | 51-085-0003 | Hanover Co. | 37.60613 -77.21880 |
| 75-B | O3, NO2, SO2, PM2.5 | Charles City County Route 608 | 51-036-0002 | Charles City Co. | 37.34438 -77.25925 |
| 154-M | PM10, Toxics | Carter G. Woodson Middle School 1000 Winston Churchill Dr. | 51-670-0010 | Hopewell | 37.28962 -77.29182 |
| 158-X | CO, NO2, PM2.5 cont. | Joseph Bryan Park | 51-760-0025 | Richmond | 37.59088 -77.46925 |

Contact Information for this Region:

Piedmont Regional Office

James Golden, Director

4949-A Cox Road

Glen Allen, VA 23060

(804) 527-5053

Tidewater Monitoring Network

| STATION NUMBER | POLLUT. | LOCATION | EPA ID | CITY/COUNTY | LAT/LONG |
| --- | --- | --- | --- | --- | --- |
| 179-K | CO, SO2, NO2, O3, PM2.5, PM2.5 cont., PM10 | NASA Langley Research Center | 51-650-0008 | Hampton | 37.10373 -76.38702 |
| 181-A1 | CO, SO2, NO2, PM10, PM2.5 | NOAA Property 2nd and Woodis Avenue | 51-710-0024 | Norfolk | 36.85555 -76.30135 |
| 183-E | O3 | Tidewater Community College Frederick Campus | 51-800-0004 | Suffolk | 36.90118 -76.43808 |
| 183-F | O3 | Tidewater Research Station | 51-800-0005 | Suffolk | 36.66525 -76.73078 |
| 184-J | PM2.5, Toxics | DEQ – Tidewater Regional Office 5636 Southern Blvd. | 51-810-0008 | Va. Beach | 36.84188 -76.18123 |

Contact information for this Region:

Craig Nicol, Director

5636 Southern Blvd.

Virginia Beach, VA 23462

(757) 518-2173

Northern Monitoring Network

| STATION NUMBER | POLLUT. | LOCATION | EPA ID | CITY/COUNTY | LAT/LONG |
| --- | --- | --- | --- | --- | --- |
| 37-B | O3 | Phelps Wildlife Area Route 651 | 51-061-0002 | Sumerduck Fauquier Co. | 38.47367 -77.76772 |
| 38-I | O3, NO2, PM2.5 | Broad Run High School Route 641 | 51-107-1005 | Ashburn Loudoun Co. | 39.02473 -77.48925 |
| 44-A | O3 | Widewater Elementary School Den Rich Road | 51-179-0001 | Widewater Stafford Co. | 38.48123 -77.37040 |
| 45-L | O3, NO2 | Long Park Route 15 | 51-153-0009 | Prince William Co. | 38.85287 -77.63462 |
| 46-B9 | SO2, O3, PM2.5, PM10 | Lee District Park Telegraph Road | 51-059-0030 | Franconia Fairfax Co. | 38.77335 -77.10468 |
| 46-C2 | CO, NO2, PM2.5 cont. | 6831 Backlick Road | 51-059-0031 | Springfield Fairfax Co. | 38.76835 -77.18347 |
| 47-T | CO, NO2, O3, PM2.5 | Aurora Hills Visitors Center 18th and Hayes Streets | 51-013-0020 | Arlington Co. | 38.85770 -77.05922 |
| 48-A | O3 | U.S.G.S. Geomagnetic Center | 51-033-0001 | Corbin Caroline Co. | 38.20087 -77.37742 |
| L-126-H | PM10 | 435 Ferdinand Day Drive | 51-510-0020 | Alexandria | 38.80493 -77.12687 |
| N-35-A | O3, TEOM, IMPROVE | Big Meadows, National Park Service | 51-113-0003 | Madison Co. | 38.52280 -78.43487 |

Contact Information for this Region:

Northern Regional Office

Thomas Faha, Director

13901 Crown Court

Woodbridge, VA 22193

(703) 583-3810

Data Capture Criteria

| Minimum Number of Observations |  |
| --- | --- |
| 3-Hour Average | 3 Consecutive Hourly Observations |
| 8-Hour | 6 Hourly Observations |
| 24-Hour | 18 Hourly Observations |
| Quarterly Averages & 3-month Averages | 75% of Scheduled Samples |
| Yearly Averages | 75% of Total Possible Observations |
| Annual Weighted Means | Four Complete Quarterly Averages |

National Ambient Air Quality Standards

| Pollutant |  | Primary/ Secondary | Averaging Time | Level | Form |
| --- | --- | --- | --- | --- | --- |
| Carbon Monoxide |  | Primary | 8-hour | 9 ppm | Not to be exceeded more than once per year |
|  |  |  | 1-hour | 35 ppm |  |
| Lead |  | Primary and Secondary | Rolling 3 month average | 0.15 μg/m3 (a) | Not to be exceeded |
| Nitrogen Dioxide |  | Primary | 1-hour | 100 ppb | 98th percentile of 1-hour daily maximum concentrations, averaged over 3 years |
|  |  | Primary and Secondary | Annual | 53 ppb (b) | Annual Mean |
| Ozone |  | Primary and Secondary | 8-hour | 0.070 ppm (c) | Annual fourth-highest daily maximum 8-hr concentration, averaged over 3 years |
| Particle Pollution | PM2.5 | Primary | Annual | 12.0 μg/m3 | Annual Mean, averaged over 3 years |
|  |  | Secondary | Annual | 15.0 μg/m3 | Annual Mean, averaged over 3 years |
|  |  | Primary and Secondary | 24-hour | 35 μg/m3 | 98th percentile, averaged over 3 years |
|  | PM10 | Primary and Secondary | 24-hour | 150 μg/m3 | Not to be exceeded more than once per year on average over 3 years |
| Sulfur Dioxide |  | Primary | 1-hour | 75 ppb (d) | 99th percentile of 1-hour daily maximum concentrations, averaged over 3 years |
|  |  | Secondary | 3-hour | 0.5 ppm | Not to be exceeded more than once per year |
| (a) In areas designated nonattainment for the Pb standards prior to the promulgation of the current (2008) standards, and for which implementation plans to attain or maintain the current (2008) standards have not been submitted and approved, the previous standards (1.5 µg/m3 as a calendar quarter average) also remain in effect. (b) The level of the annual NO2 standard is 0.053 ppm. It is shown here in terms of ppb for the purposes of clearer comparison to the 1-hour standard level. (c) Final rule signed October 1, 2015, and effective December 28, 2015. The previous (2008) O3 standards additionally remain in effect in some areas. Revocation of the previous (2008) O3 standards and transitioning to the current (2015) standards will be addressed in the implementation rule for the current standards. (d) The previous SO2 standards (0.14 ppm 24-hour and 0.03 ppm annual) will additionally remain in effect in certain areas: (1) any area for which it is not yet 1 year since the effective date of designation under the current (2010) standards, and (2) any area for which implementation plans providing for attainment of the current (2010) standard have not been submitted and approved and which is designated nonattainment under the previous SO2 standards or is not meeting the requirements of a SIP call under the previous SO2 standards (40 CFR 50.4(3)). A SIP call is an EPA action requiring a state to resubmit all or part of its State Implementation Plan to demonstrate attainment of the required NAAQS. |  |  |  |  |  |

Please see EPA's NAAQS Table for additional information.

Number of Criteria Pollutant Monitoring Sites

NCore/SLAMS 2019

| REGION | PM2.5 | PM10 | Pb | CO | SO2 | NO2 | O3 | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Southwest | 1 | 1 | --- | --- | --- | --- | 1 | 3 |
| Valley | 3 | 1 | --- | --- | 1 | 1 | 4 | 10 |
| Blue Ridge | 3 | --- | 2 | 1 | 1 | 1 | 1 | 9 |
| Piedmont | 5 | 2 | --- | 2 | 2 | 3 | 4 | 18 |
| Tidewater | 3 | 2 | --- | 2 | 2 | 2 | 3 | 14 |
| *Northern | 4 | 3 | --- | 2 | 1 | 4 | 8 | 22 |
| TOTAL | 19 | 9 | 2 | 7 | 7 | 11 | 21 | 76 |

* This region’s sites are operated by DEQ, Alexandria Health Dept., and NPS

EPA is responsible for designating nonattainment and attainment/maintenance areas for the Commonwealth.  Nonattainment and maintenance areas generally correspond to VOC and NOX emissions control areas.  Virginia implements additional federal nonattainment and maintenance requirements within these control areas.  Additionally, Virginia may designate certain areas of the state as VOC and NOX emissions control areas in order to implement requirements as needed to protect air quality.  A list of VOC and NOX emissions control areas is found in the Regulations for the Control and Abatement of Air Pollution at 9VAC5-20-206.

Air Quality Planning Areas/Nonattainment for Virginia

Appendix B

Air Quality Internet Links

AirData – Access to national and state air pollution data

Outdoor Air Quality Data

Air Emission Data Sources

Emissions Sources

Air Now – AQI maps

Real Time AQI

American Lung Association:

USA Lung

AQS Data Mart (AQS data for the scientific and technical community):

Data Mart

CASTNET (Clean Air Status and Trends Network)

Castnet

Department of Environmental Quality link:

Virginia DEQ

Environmental Education for teachers and students:

Resources for teachers

DEQ Education webpage

EPA Popular Resources

Environmental Topics

EPA’s Technology Transfer Network (TTN) – Ambient Monitoring Technology Information Center (AMTIC)

IMPROVE

Interagency Monitoring of Protected Visual Environments

MARAMA

Mid-Atlantic Regional Air Management Association

Nonattainment area descriptions:

EPA's Green Book

U.S. EPA:

EPA

2019 3-Day Monitoring Schedule for PM2.5, PM10, TSP and VOCs

Sampling Schedule Calendar

## National Air Quality: Status and Trends of Key Air Pollutants

Air Trends

References

Code of Federal Regulations – 40 CFR 50 & 58

Electronic Code of Federal Regulations

Virginia Ambient Air Monitoring Data Reports

DEQ's Air Monitoring Publications

Air Quality System (AQS)

EPA's AQS

National Ambient Air Quality Standards

EPA NAAQS